NUTURESENSE

Novelty, Research Problem & Proposed Technical Contribution

Secure Heterogeneous Sensing over Constrained Long-Range LoRa Links

1. Proposed Novelty

Primary novelty statement A common security abstraction for protecting heterogeneous sensing data over constrained LoRa links, supporting multiple standardized AEAD schemes and authenticated, synchronized adaptive transitions between them.

## What this means technically

Heterogeneous sensors are decoupled from the choice of cryptographic primitive.

A common security interface accepts sensing data plus required metadata/context and returns a protected packet.

Multiple standardized AEAD schemes can be plugged into the abstraction, initially AES-GCM and ChaCha20-Poly1305.

The security controller manages algorithm state, key/nonce state, replay state, and policy rather than individual sensors implementing cryptography.

Algorithm changes are treated as authenticated protocol state transitions, not unauthenticated local decisions.

Nonce/key state, replay state, sender/receiver synchronization, and downgrade resistance are first-class protocol requirements.

The contribution is experimentally evaluated on the constrained communication pipeline.

## Research-contribution formulation

NutureSense investigates whether authenticated cryptographic agility can be introduced into a heterogeneous IoT sensing pipeline over constrained LoRa communication without compromising cryptographic synchronization, nonce/key safety, replay resistance, downgrade protection, or communication efficiency.

# 2. Research Problem

Core problem How can heterogeneous sensing data be protected over a constrained LoRa link through a common security abstraction that supports multiple AEAD schemes, while ensuring that adaptive cryptographic transitions are authenticated, synchronized, replay-resistant, and efficient enough for constrained communication?

## Why this is a real systems-security problem

Different sensing sources should not need separate cryptographic implementations.

Constrained links make packet overhead, latency, processing cost, and energy important security metrics.

AEAD provides confidentiality and integrity/authenticity, but replay protection and protocol-state management remain separate requirements.

If sender and receiver change cryptographic state inconsistently, legitimate packets can fail or an attacker may attempt desynchronization.

Adaptive mechanisms create control/state-transition messages that themselves require authentication and replay protection.

The usefulness of adaptation must be demonstrated experimentally rather than assumed.

# 3. Research Questions

RQ1  Can heterogeneous sensor data be protected through one common security abstraction without coupling sensing modules to a specific AEAD implementation?

RQ2  Can sender and receiver transition between AEAD states using an authenticated and synchronized protocol without accepting forged, replayed, or unauthorized transition requests?

RQ3  How should key state, nonce uniqueness, sequence state, and replay state be maintained across cryptographic transitions?

RQ4  What security and performance overhead is introduced by cryptographic agility compared with fixed AES-GCM and fixed ChaCha20-Poly1305?

RQ5  Under packet loss, duplication, and out-of-order delivery, can the protocol maintain correct security state without unnecessary desynchronization?

RQ6  Does adaptive selection provide a measurable benefit under the selected policy, or is a fixed AEAD scheme preferable?

# 4. Flowchart — Overall Architecture

HETEROGENEOUS SENSING RuView / Wi-Fi CSI • Temperature • Other Sensors

↓

COMMON SENSOR DATA MODEL Normalize payload + metadata/context

↓

COMMON SECURITY ABSTRACTION protect() • verify() • transition()

↓

SECURITY CONTROLLER Policy • Key/Nonce State • Replay State • Algorithm State

↓

| AES-GCM | OR | ChaCha20-Poly1305 |
| --- | --- | --- |

↓

PROTECTED PACKET → CONSTRAINED LONG-RANGE LoRa

↓

RECEIVER SECURITY PROCESSING Authenticate • Replay check • State check • Decrypt

↓

ORIGINAL SENSING INFORMATION Accepted / Rejected + Security Event Log

# 5. Flowchart — Authenticated Cryptographic State Transition

CURRENT STATE Example: AES-GCM

↓

SECURITY POLICY / TRIGGER Request a legitimate state transition

↓

AUTHENTICATED TRANSITION REQUEST Current state • New state • Transition counter • Nonce • Authentication

↓

RECEIVER VALIDATION Verify sender • Verify authentication • Check replay/counter • Check allowed transition

↓

| VALID | → | INVALID / REPLAYED / UNAUTHORIZED |
| --- | --- | --- |

↓

VALID → BOTH SIDES COMMIT TO NEW STATE Update algorithm/key/nonce/replay state consistently

↓

NEW AEAD STATE Example: ChaCha20-Poly1305

# 6. Flowchart — Attack Evaluation

LEGITIMATE / ADVERSARIAL TRAFFIC

↓

| Tampering | Replay | Forgery | Unauthorized transition |
| --- | --- | --- | --- |

↓

RECEIVER SECURITY CONTROLS AEAD verification • Replay/sequence validation • State validation

↓

DECISION ACCEPT legitimate packet ¦ REJECT attack ¦ LOG security event

# 7. Conceptual Protected Packet

| Header | Sender ID | Sequence | Nonce | Ciphertext | Auth Tag |
| --- | --- | --- | --- | --- | --- |

Header: protocol/version information.

Sender ID: identifies the source and can be cryptographically bound where appropriate.

Sequence number: supports replay/out-of-order policy.

Nonce: must remain unique under the relevant key; a sequence number alone may not guarantee uniqueness across reboot/session changes.

Ciphertext: protected sensing payload.

Authentication tag: AEAD integrity/authenticity result.

# 8. Evaluation Plan

| Configuration | Security tests | Performance metrics | Purpose |
| --- | --- | --- | --- |
| Plain baseline | Link baseline | PDR, latency, throughput | Reference |
| AES-GCM | Tamper, replay, forgery | CPU, memory, latency, overhead | Fixed-AEAD baseline |
| ChaCha20-Poly1305 | Tamper, replay, forgery | CPU, memory, latency, overhead | Second AEAD baseline |
| Adaptive protocol | All attacks + transition attacks | Transition cost, PDR, latency, resources | Test the proposed contribution |

# 9. What Is and Is Not Novel

| Component | Assessment | Position in NutureSense |
| --- | --- | --- |
| LoRa security | Established | Communication environment |
| AES-GCM / ChaCha20-Poly1305 | Established | Standard AEAD baselines |
| Adaptive/selective encryption | Crowded | Enabling mechanism, not headline novelty |
| Common security abstraction | Potential contribution | Core architecture |
| Authenticated state transitions | Potential contribution | Core protocol problem |
| Synchronized transitions + replay/downgrade protection | Strongest candidate | Core security mechanism |
| Empirical evaluation on constrained LoRa | Research contribution | Validation and evidence |

# 10. Expected Contributions

A sensor-agnostic common security abstraction for heterogeneous sensing data.

A modular AEAD layer supporting multiple standardized constructions.

An authenticated and synchronized cryptographic state-transition mechanism.

Explicit handling of nonce/key state, replay state, and transition authorization.

A controlled attack suite covering packet tampering, replay, forgery, and unauthorized transitions.

Empirical comparison between fixed and adaptive security configurations on constrained communication.

A reusable protocol/architecture that can later support additional sensing sources or communication configurations.

# 11. Important Boundaries

Do not claim NutureSense invents AES-GCM, ChaCha20-Poly1305, LoRa security, or adaptive encryption.

Do not assume adaptive selection is automatically more secure or efficient.

Do not claim medical diagnosis, prediction, or prevention of SIDS.

Do not claim patentability from this document; formal patentability requires claim-level prior-art analysis and professional legal review.

Exact adaptive trigger, LoRa architecture, key provisioning, hardware, and sensor set remain faculty/hardware-dependent until confirmed.

# 12. Research Context

NIST's IoT cybersecurity capability catalog identifies device identification, data protection, software update, cybersecurity state awareness, and device security as core technical capabilities. This supports treating IoT security as a system capability rather than reducing the project to encryption alone. Recent 2026 research also demonstrates adaptive authenticated encryption in LoRaWAN environments, reinforcing the need to differentiate NutureSense through its common security abstraction, authenticated/synchronized state transitions, heterogeneous sensing integration, and empirical evaluation.

References: NIST IoT Device Cybersecurity Capabilities Catalog; NIST SP 800-213; KR102104703B1; Abinaya et al., Discover Internet of Things (2026), “Deep reinforcement learning-optimized adaptive authenticated encryption and dynamic transmission control for secure LoRaWAN IoT communication.”